LABOUR CALCULATION DETAIL

INTERNAL ONLY

| Client | Linda Bushel | Project | Bushel Rear Deck (pool deck) |
| --- | --- | --- | --- |
| Site | 12 D'Arcy's Way, Kemptville, ON K0G 1J0 | Date | 2026-10-27 |

Direct labour hours, rates, and quoted totals are project-specific and are not stored in this reusable master.

| Labour Task | Basis / Scope | Hours | Direct labour | GM reference sell |
| --- | --- | --- | --- | --- |
| Layout and mark the pile locations | deck and stair lines; 11 pile locations marked for Pro Post (Pro Post supplies AND installs the piles: no pile labour here) | 4 | $300.00 | $342.86 |
| Posts and beams | 11 posts cut on brackets; built-up 2×10 beams (7 × 12 ft + 4 × 10 ft), 3/8 in carriage bolts | 16 | $1,200.00 | $1,371.43 |
| Upper-deck framing | 16 joists cut to the pool curve, 8-piece curved header with 14 skewed hangers, blocking | 24 | $1,800.00 | $2,057.14 |
| Lower-deck framing | 9 joists 3 ft, lower beam, blocking, lower posts cut on site | 8 | $600.00 | $685.71 |
| Stairs | 10 ft wide, 4 treads, 5 risers; stringers cut and hung | 16 | $1,200.00 | $1,371.43 |
| Trex decking | about 147 sq ft (upper 116.8 + lower 30), every board scribed to the pool curve on the upper deck | 16 | $1,200.00 | $1,371.43 |
| Stair treads and risers (Trex) | 8 tread boards, 5 riser boards 10 ft | 8 | $600.00 | $685.71 |
| Trex fascia | about 44 ft of exposed rim, colour-matched screws | 5 | $375.00 | $428.57 |
| Railing blocking | solid blocking under each Vista post base | 4 | $300.00 | $342.86 |
| Vista railing and gate | 11 sections, 14 posts on base plates, pickets, 48 in gate | 16 | $1,200.00 | $1,371.43 |
| Unload and carry materials | BMR delivery to the back yard (carrying is extra from BMR) | 4 | $300.00 | $342.86 |
| Clean-up and final walk-through | debris, offcuts, punch list | 4 | $300.00 | $342.86 |
| Total | Two-person crew | 125 | $9,375.00 | $10,714.29 |

# Labour Control

| Metric | Amount |
| --- | --- |
| Total man-hours | 125 |
| Direct labour rate | $75.00 per man-hour (Brayman rate from 27 Oct 2026) |
| Direct labour | $9,375.00 |
| GM reference sell (12.5% true gross margin, Linda project decision) | $10,714.29 |
| Crew and duration | Two-person crew, about 7.8 working days |
| Helical piles | Supplied and installed by Pro Post Foundations (OT-2236); no pile installation in Brayman labour |
| Basis | Joel's estimating allowance (27 Oct 2026) from the J1 take-off and supplier quantities |
| For Ben | Review each line; adjust the hours of any task from your experience and the price is recalculated |

Direct labour hours and rates are project-specific and are not stored in this reusable master.